Wave Release Coordination under Vertical Resource Constraints in Multi-Story AMR Warehouse

Abstract

In multi-story fulfillment warehouses, autonomous mobile robots (AMRs) operate on individual floors and use shared freight elevators for inter-floor transfers. Elevator capacity can become the primary source of delay. Before dispatch begins, wave composition, the choice of which orders to release together, shapes the number, direction, and timing of elevator requests and makespan. To our knowledge, no formulation treats it as a decision variable coupled with shared vertical transport. We formulate wave-release coordination under vertical resource constraints, holding the downstream operational dispatch policy fixed. Each wave is represented by vertical spread, directional imbalance, and temporal clustering. The Bound-and-Gap decomposition partitions composition-related variation in makespan into a structural upper bound that a policy cannot recover at the selected representation resolution and recoverable policy slack; finer wave-profile grouping can only increase the upper bound. Under elevator-model uncertainty, the Model-Dominance Hedge Rule selects the wave-profile class with the smallest makespan under the conservative co-occupancy batching model. Under stated chain-dominance conditions, it minimizes worst-case makespan and coincides with a calibrated distributionally robust solution. In 106,800 pre-registered simulations, dominance holds in 99.2% of matched wave pairs, and the Hedge Rule matches the distributionally robust solution in all six configurations. The decomposition identities hold in all 72 cells, whereas the resolution check is inconclusive only when estimated gaps are small. Across 18 configurations, the capacity-versus-policy conclusion is stable across random seeds and unanimous in 15 of 18 configurations. The tools help operators decide whether to improve wave composition or expand elevator capacity.

1 Introduction

Urban land constraints and increasingly short fulfillment promises are encouraging the development of multi-story fulfillment warehouses. In these facilities, autonomous mobile robots (AMRs) move efficiently within each floor, but a limited number of shared freight elevators carry all inter-floor traffic. When vertical requests arrive in concentrated bursts, elevator queues delay AMRs and propagate through picking, replenishment, and order completion. Warehouse performance therefore depends not only on how AMRs and elevators are dispatched after work is released, but also on how that work is created before dispatch begins.

A central upstream decision is wave composition: which orders are released together within a wave. A wave determines the floors to be served, the directions and timing of elevator requests, and the degree of concurrent demand imposed on the fleet. It consequently shapes the elevator-trip distribution before any AMR moves. In practice, operators may form waves by arrival order, destination floor, or carrier cutoff. Revising those groupings before release changes neither the warehouse layout nor fleet size, so it is a low-capital way to manage vertical congestion. The operational value of recomposing waves depends on the source of congestion. When elevator capacity is binding, recomposing waves has limited scope to reduce delay. When congestion stems from the pattern of elevator requests, however, a better wave composition can reduce avoidable queueing. Operators therefore need to distinguish these two regimes before choosing between a revised wave-release policy and additional elevator capacity.

Existing studies address important parts of this setting. Order-batching research determines which orders are processed together, multi-story robotic fulfillment studies model vertical transport, and elevator-control studies optimize dispatch under congestion. However, these streams generally fix either the order set, wave content, or transport domain. The unresolved upstream decision is wave composition in a system where a flexible, floor-bound AMR fleet competes for shared freight elevators. Because wave composition shapes the elevator-trip distribution before dispatch begins, it cannot be optimized independently of the vertical bottleneck. Section 2 positions this problem relative to the relevant literature.

We study wave-release coordination as a two-stage decision problem. At the tactical stage, the operator composes a wave; at the operational stage, a fixed operational dispatch policy assigns AMRs and manages elevator trips. We represent each wave by a three-feature wave profile, : vertical spread, directional imbalance, and temporal clustering. This representation supports two tasks: quantifying the variation in makespan that a better wave-release policy can recover and selecting a wave-profile class under elevator-model uncertainty. The Bound-and-Gap decomposition addresses the first task, and the Model-Dominance Hedge Rule addresses the second. Formal definitions, assumptions, and proofs are provided in Sections 3 and 4.

This paper makes three contributions. First, it formalizes the coupled upstream decision involving wave composition, multi-story deployment, a flexible AMR fleet, and shared-elevator capacity. Related studies typically fix wave content or request sets, or confine transport to a single device or tier; our formulation models the coupling explicitly.

Second, we develop two complementary analytical tools for wave-release coordination. The Bound-and-Gap decomposition is diagnostic: it quantifies how much composition-related performance variation is structurally constrained and how much a better wave-release policy can recover. Formally, it separates  into a structural upper bound, , that a wave-release policy cannot recover at the selected  resolution and recoverable policy slack, . We show that  equals the predict-then-optimize decision loss, thereby linking the slack to existing decision-focused methods. A resolution theorem shows that refining the wave-profile grouping can only increase . The Model-Dominance Hedge Rule is prescriptive: it selects a robust wave-profile class when the elevator model is uncertain. Under stated dominance conditions, the minimax wave-profile-class selection reduces to the class that is optimal under the conservative co-occupancy batching model and coincides with a calibrated distributionally robust solution. When dominance is only approximate, the analysis provides a computable worst-case-loss bound.

Third, we provide an empirical capacity-versus-policy diagnosis that identifies whether, in a given operating regime, a revised wave-release policy or additional elevator capacity offers the more effective intervention. We evaluate this diagnosis in a pre-registered study of 18 configurations and 106,800 simulations. Under the pre-registered wave grouping and median metric, elevator capacity is the binding constraint in the evaluated settings: approximately 72% of the mean composition-related gap is structural, leaving approximately 28% as recoverable policy slack. The capacity-versus-policy conclusion is stable across random seeds and unanimous in 15 of the 18 configurations. Across the six configurations that directly compare the candidate models, chain dominance holds in 99.2% of matched wave pairs, never falls below 96.0% in an individual cell, and persists as elevator capacity varies from two to five loads per trip. The Hedge Rule selects the same wave-profile class as the calibrated distributionally robust solution in all six configurations. The decomposition identities hold exactly in all 72 experimental cells. Its statistical test detects a nonzero composition-related effect in 57 of the 72 cells; the remaining cells are inconclusive because the estimated effects are small. Together, these results provide a regime-specific basis for selecting between demand shaping through wave composition and capacity expansion.

The remainder of the paper is organized as follows. Section 2 reviews related work. Section 3 formulates the problem, Section 4 develops the analytical tools, Section 5 presents the computational study, and Section 6 concludes.

2 Related works

Relevant literature for this study comprises three operational streams: wave composition, vertical transport, and downstream dispatch, and two methodological streams: decision loss and information refinement, and model uncertainty. The review first delineates the scope of the three operational streams and then discusses the methodological foundations for the two analytical tools used in this study. Across these streams, the upstream coupling between wave composition and shared, multi-story elevator capacity appears to receive limited direct attention. Recent surveys described the broader warehouse landscape, including the evolution of warehousing research, e-commerce fulfillment, robotized warehouse systems, and order-batching problems (Azadeh et al., 2019; Boysen et al., 2019; Boysen & de Koster, 2025; Pardo et al., 2024)

First, warehouse-operations research examined wave composition as a decision problem but did not model its consequences for shared-elevator demand in a flexible AMR fleet. Representative studies addressed wave picking and walk-and-pick systems (Bartholdi & Hackman, 2019; Bozer & Kile, 2008; Gademann et al., 2001), multiple-picker makespan minimization (Ardjmand et al., 2018), and e-commerce order-line batching and scheduling (Haouassi et al., 2022; Rasmi et al., 2022; Schiffer et al., 2022). AMR-assisted studies jointly considered order batching, assignment, and routing (Scholz et al., 2017; Žulj et al., 2022), while Qin et al. (2024) examined wave cardinality in a multi-tote AMR system. Device-level work considered batching across vertical lift modules and order-spread sequencing in automated sorting (Boysen et al., 2018; Lenoble et al., 2018), and recent robotic mobile fulfillment system (RMFS) batching work focused primarily on single-tier throughput (e.g., Liu et al. (2025)). These studies provided important foundations for wave composition, but they did not address how it changes demand for elevators shared by a flexible AMR fleet across multiple floors. In contrast, this study represents wave composition with a wave profile, , whose features describe the resulting vertical-trip demand.

Second, research on multi-story robotic mobile fulfillment systems (RMFSs) and multi-tier shuttle systems modeled vertical transport, but typically treated the task set as given before operational decisions were optimized. Examples included inbound-job scheduling in four-way shuttle systems (Z. Wu et al., 2024), order and pod assignment in multi-level RMFSs (Tadumadze et al., 2023), queueing models for RMFSs and compact storage systems with lifts (Lamballais et al., 2017; Tappia et al., 2015), and retrieval scheduling under two lifts (Chen et al., 2023). J. Wu et al. (2025) jointly optimized picking and replenishment within a tier. Wang et al. (2025) provided a closely related exception by jointly scheduling retrieval requests, shuttles, and two shared lift types in a multi-tier shuttle system. Nevertheless, their retrieval-request set was specified in advance. The present study instead treats wave composition as the upstream tactical decision that shapes vertical congestion before a fixed operational dispatch policy is applied.

Third, research closest to the present physical setting optimized elevator or robot operations under exogenous demand rather than altering that demand through wave composition. Chakravarty et al. (2025) formulated multi-robot lift scheduling with Boolean satisfiability; Tsai et al. (2025) optimized elevator standby strategies; and Richer et al. (2024) studied static elevator dispatching with destination control. A parallel literature used reinforcement learning for downstream scheduling and control, from elevator group control (Crites et al., 1998) to traffic-aware elevator dispatching (Wan et al., 2024), batch-order scheduling in RMFSs (Cheng et al., 2024), multi-robot task allocation (Dhanaraj et al., 2025; Ma et al., 2025; Wen & Ma, 2024), and AMR-fleet control (Wesselhöft et al., 2022). These approaches learned or optimized an operational dispatch policy after demand was given, but the present framework changes the demand pattern through wave composition. The approaches are therefore complementary.

The Bound-and-Gap decomposition draws on a methodological stream that examined decision loss and information refinement. Predict-then-optimize research examined the loss incurred when a learned prediction drove a downstream optimization decision (Adam N. Elmachtoub & Paul Grigas, 2021; Chenreddy & Delage, 2024; Vera et al., 2020), while decision-focused learning and generalization studies characterized how such loss could be reduced (Balghiti et al., 2022; Mandi et al., 2024). The Bound-and-Gap decomposition connects this decision-loss perspective to recoverable policy slack in wave composition. Its representation-resolution component is also related to classical work on information refinement and partition-based stochastic optimization (Blackwell, 1953; Song & Luedtke, 2015): a more refined wave-profile class can reveal additional decision-relevant variation.

The Model-Dominance Hedge Rule draws on a complementary methodological stream that addressed decisions under model uncertainty. Wasserstein distributionally robust optimization addresses uncertainty by optimizing against adverse probability distributions within a specified ambiguity set (Blanchet & Murthy, 2019; Gao & Kleywegt, 2023; Mohajerin Esfahani & Kuhn, 2018), building on broader robust-optimization foundations (Bertsimas et al., 2011; Delage & Ye, 2010). Sadana et al. (2025) placed these approaches within the broader contextual-optimization literature. The Model-Dominance Hedge Rule draws on this perspective to select a wave-profile class when the elevator model is uncertain. Finally, robust scheduling under model uncertainty usually addresses parametric variation within an agreed model family (Lu & Shen, 2021; Wiesemann et al., 2013). Here, the uncertainty is structural: the throughput abstraction and the conservative co-occupancy batching model represent different ways of modelling elevator trips. This distinction motivates a rule based on model dominance rather than parameter estimation.

These streams leave unresolved the upstream coupling between wave composition and shared vertical capacity. This study integrates these components by treating wave composition, multi-story deployment, flexible AMR fleets, and shared elevator capacity as one coupled decision setting. It thereby complements downstream scheduling methods with an upstream, interpretable basis for deciding when and how to modify the demand entering the vertical transport system.

Problem Formulation

We consider a multi-story fulfillment warehouse with F floors connected by E shared freight elevators. A fleet of ∣A∣ floor-bound autonomous mobile robots (AMRs) handle horizontal travel on each floor; floor-to-floor transfers go through the elevators, with each elevator carrying at most c AMRs per trip. Orders arrive over a planning horizon, each with a known source floor, destination floor, and release time. A wave is a subset of orders released into the system simultaneously at the start of a release window of length Δ. The system performance metric is the wave makespan: the time at which the last order in the wave is delivered, given a fixed operational dispatch policy.

This setting decomposes naturally into two coupled decision layers. The tactical layer chooses which orders to bundle into a wave W, namely the wave composition decision; this is the focus of our analysis. The operational layer determines, given W, which AMR serves which order, which AMRs board each elevator trip, and in what order. We delegate these decisions to a deterministic dispatch policy, defaulting to capacity-bounded FIFO boarding under the elevator model M∈M, with full simulator details given in Section 4. We model only the wave-composition decision as a formal variable; downstream AMR-elevator execution is a simulator-realized policy. This simplification is deliberate: the wave-elevator coupling is fully captured by Φ(W) at the partition resolution on which our analytical tools operate; finer partitions reveal additional structure within Φ-cells, a property formalized as the refinement monotonicity result of Corollary 1 in Section 4. Operational lift scheduling is well studied (Chakravarty et al., 2025); the upstream wave layer that prior work treats as exogenous is formalized in this paper.

Sets and indexes

O: set of all orders over the planning horizon, indexed by o.

F={1, 2, .., F}: set of floors, indexed by f.

A: AMR fleet, indexed by a.

E: set of elevators, indexed by e.

Wω: sequence of waves over the planning horizon, indexed by ω.

M=M1, M2, M3: elevator-model set, indexed by m.

Parameters

All parameters below are deterministic under elevator models M1 and M2. Under M3, the timing primitives (τs, τe, τd) become lognormally perturbed with scale σ{M3}∈{0.10, 0.20}. Numerical values are reported in Section 5.

Warehouse and fleet:

F∈Z≥2: number of floors

A: AMR fleet size

E: number of elevators

c∈Z≥1: per-trip elevator capacity

Timing primitives:

τs∈R>0: per-event AMR service time at pickup or dropoff

τe∈R>0: per-floor elevator travel time

τd∈R>0: elevator dwell time per stop

Operator-set release controls:

Wmin, Wmax∈Z≥1: wave cardinality bounds

τω∈R>0: release window start of wave ω

∆∈R>0: release window length

Order attributes (input data):

so∈F: source floor of order o

do∈F: destination floor of order o (same-floor orders with do=so are admissible; they contribute no elevator demand and are excluded from the IW computation)

ro∈R≥0: release time of order o

Decision variable

The model has a single decision variable, the tactical wave composition.

xo∈0, 1, ∀o∈O: wave-inclusion indicator, with xo=1 if order o is released in the current wave.

The wave composition is the induced set W={o∈O: xo=1}.

Assumptions

The formulation rests on five assumptions, each stated explicitly, so the scope of validity is identifiable.

A1. Single-order AMR carriage. Each AMR transports at most one order at a time. This aligns with industry practice for parcel-scale fulfillment and rules out multi-tote AMR variants(Qin et al., 2024).

A2. Source-before-destination sequencing. For each served order o, the assigned AMR visits the source floor so before the destination floor do.

A3. Capacity-bounded FIFO boarding. AMRs join the elevator queue in arrival order and board up to c at a time under the elevator model M.

A4. Intra-floor travel collapsed into service time. AMR horizontal travel within a floor is summarized by the per-event service time τs, with no explicit congestion or routing on the floor.

A5. Elevator model is a fixed property of the warehouse, unknown to the operator at decision time. The elevator model M∈M is a structural property of the physical elevator subsystem (its boarding logic and trip-time aggregation), exogenous to the wave-composition decision. Because the warehouse OR literature uses both throughput-aggregation (Bartholdi & Hackman, 2019) and co-occupancy batching (Chakravarty et al., 2025), we do not commit to a single convention.

Structured-feature representation

We summarize each candidate wave by a three-dimensional fingerprint ΦW=(CW, IW, T(W)), a function of the decision {xo}, on which the analytical tools of Section 4 operate. The three axes each capture a distinct physical mechanism of vertical resource stress.

- Vertical activity diversity CW≥0. Shannon entropy (natural logarithm) of the empirical floor distribution taken over the union of source and destination floors of orders in W. With pfW=o∈W:so=f+o∈W:do=f2W, we define CW=-f∈FpfWlnpf(W), with 0ln0=0. High C indicates that orders span many floors (diversified vertical activity); low C indicates concentration on a few floors.

- Directional imbalance I(W)∈[0, 1]: It measures how one-directional the cross-floor traffic is. With W↑=o∈W:do>so and W↓=o∈W:do<so (same-floor orders excluded from both), IW=W↑-W↓W↑+W↓ when W↑+W↓>0, and IW=0 otherwise.

- Temporal clustering T(W)≥0: It is the coefficient of variation of per-order release times, measured relative to the wave’s release-window start. TW=σr(W)μr(W). High T means orders arrive in tight bursts; low T means they spread evenly across the window.

The three axes capture conceptually distinct mechanisms: multi-floor spread, up-versus-down asymmetry, and temporal bunching, each with a clear physical meaning. We treat Φ as a conceptual decomposition rather than a predictive surrogate.

Elevator models

The dispatch policy is realized through a simulator under one of three elevator models:

- M1(throughput aggregation): Each elevator is modeled as a server with a per-AMR throughput rate that incorporates batched-trip efficiency into a single multiplier.

- M2(true co-occupancy batching): At each elevator, the next c AMRs in the queue board together and ride as a unit, with realistic per-trip capacity, dwell time, and direction handling.

- M3 (stochastic batching): It extends M2 with lognormal noise on per-trip duration at scale σM3, used for robustness sensitivity.

M1 and M2 are both well-attested in the warehouse OR literature; the structural disagreement between throughput aggregation and explicit co-occupancy is the methodological gap our Hedge Rule resolves.

Objective function

The objective function minimizes the expected wave makespan under the chosen elevator model:

| minW⊆OE[Cmax(W;M, ξ)] | (1) |
| --- | --- |

where CmaxW;M, ξ≔maxo∈Wtodeliver(W;M, ξ) is the wave makespan, todeliver(W;M, ξ) is the simulator-realized delivery time of order o under elevator model M, and ξ denotes a realization of the operational randomness in the dispatch simulator. Under M1 and M2, ξ is degenerate and (1) reduces to the deterministic makespan CmaxW;M; under M3, ξ equals the lognormal perturbations on (τs, τe, τd) at scale σ{M3} and the expectation is non-degenerate. The expectation in (1) is taken over ξ. We adopt the makespan rather than the average delivery time because wave throughput in multi-story fulfillment is gated by the slowest order: the next wave cannot release until elevator capacity is freed by the current wave's completion.

Constraints

The wave composition W is subject to two classes of constraints:

| Wmin≤W≤Wmax | (2) |
| --- | --- |
| ro∈τω, τω+∆, ∀o∈W | (3) |
| ω1[o∈Wω]=1, ∀o∈O | (4) |
| xo∈0, 1, ∀o∈O | (5) |

Constraints (2)-(5) define the wave-composition decision space: cardinality bounds, temporal feasibility, exactly-one-wave-per-order, and the binary inclusion indicator. The elevator capacity bound (naboarde, t≤c) and the AMR fleet limit A are not formal optimization constraints at the wave-composition stage. They are invariants enforced by the operational simulator under the dispatch policy of Assumptions A1–A3, where the wave makespan Cmax(W;M) is realized. The capacity bound enters the formulation through its effect on Cmax (a wave with composition incompatible with c manifests as a higher Cmax(W;M) rather than as infeasibility), making it natural to absorb into the objective rather than to state as a constraint. We solve (1)-(5) one wave at a time: at each release window, the decision is over orders not yet released, so (4) is automatically satisfied. The decision object in the rest of the paper is therefore a single wave's inclusion vector.

Equations (1)-(5), together with Assumptions A1-A5, define the wave release coordination problem under vertical resource constraints. This minimal formulation is deliberate: the wave-elevator coupling is fully captured by ΦW=(CW, IW, T(W)) at the partition resolution on which our analytical tools operate. Operational lift scheduling is well studied (Chakravarty et al., 2025); the upstream wave layer, which prior work treats as exogenous, is formalized in this paper. Direct solution by enumeration over {xo}o is intractable in any case (combinatorial |O|, simulator-realized objective), so Section 4’s tools are how the formulation becomes operational.

Adam N. Elmachtoub, & Paul Grigas. (2021). Smart “Predict, then Optimize.” Management Science, 68(1), 9–26.

Ardjmand, E., Shakeri, H., Singh, M., & Sanei Bajgiran, O. (2018). Minimizing order picking makespan with multiple pickers in a wave picking warehouse. International Journal of Production Economics, 206, 169–183. https://doi.org/10.1016/j.ijpe.2018.10.001

Azadeh, K., De Koster, R., & Roy, D. (2019). Robotized and automated warehouse systems: Review and recent developments. In Transportation Science (Vol. 53, Number 4, pp. 917–945). INFORMS Inst.for Operations Res.and the Management Sciences. https://doi.org/10.1287/trsc.2018.0873

Balghiti, O. El, Elmachtoub, A. N., Grigas, P., & Tewari, A. (2022). Generalization Bounds in the Predict-then-Optimize Framework. http://arxiv.org/abs/1905.11488

Bartholdi, J. J., & Hackman, S. T. (2019). Warehouse & Distribution Science. www.warehouse-science.com

Bertsimas, D., Brown, D. B., & Caramanis, C. (2011). Theory and applications of robust optimization. In SIAM Review (Vol. 53, Number 3, pp. 464–501). https://doi.org/10.1137/080734510

Blackwell, D. (1953). Equivalent Comparisons of Experiments. The Annals of Mathematical Statistics, 24(2), 265 – 272. https://doi.org/10.1214/aoms/1177729032

Blanchet, J., & Murthy, K. (2019). Quantifying Distributional Model Risk via Optimal Transport MATHEMATICS OF OPERATIONS RESEARCH Quantifying Distributional Model Risk via Optimal Transport. Source: Mathematics of Operations Research, 44(2), 565–600. https://doi.org/10.2307/27287849

Boysen, N., & de Koster, R. (2025). 50 years of warehousing research—An operations research perspective. In European Journal of Operational Research (Vol. 320, Number 3, pp. 449–464). Elsevier B.V. https://doi.org/10.1016/j.ejor.2024.03.026

Boysen, N., de Koster, R., & Weidinger, F. (2019). Warehousing in the e-commerce era: A survey. In European Journal of Operational Research (Vol. 277, Number 2, pp. 396–411). Elsevier B.V. https://doi.org/10.1016/j.ejor.2018.08.023

Boysen, N., Fedtke, S., & Weidinger, F. (2018). Optimizing automated sorting in warehouses: The minimum order spread sequencing problem. European Journal of Operational Research, 270(1), 386–400. https://doi.org/10.1016/j.ejor.2018.03.026

Bozer, Y. A., & Kile, J. W. (2008). Order batching in walk-and-pick order picking systems. International Journal of Production Research, 46(7), 1887–1909. https://doi.org/10.1080/00207540600920850

Chakravarty, A., Grey, M. X., Muthugala, M. A. V. J., & Elara, R. M. (2025). Toward Optimal Multi-Agent Robot and Lift Schedules via Boolean Satisfiability. Mathematics, 13(18). https://doi.org/10.3390/math13183031

Chen, R., Yang, J., Yu, Y., & Guo, X. (2023). Retrieval request scheduling in a shuttle-based storage and retrieval system with two lifts. Transportation Research Part E: Logistics and Transportation Review, 174, 103117. https://doi.org/10.1016/j.tre.2023.103117

Cheng, B., Wang, L., Tan, Q., & Zhou, M. (2024). A deep reinforcement learning hyper-heuristic to solve order batching problem with mobile robots. Applied Intelligence, 54(9–10), 6865–6887. https://doi.org/10.1007/s10489-024-05532-9

Chenreddy, A., & Delage, E. (2024). End-to-end Conditional Robust Optimization. http://arxiv.org/abs/2403.04670

Crites, R. H., Barto, A. G., Huhns, M., & Weiss, G. (1998). Elevator Group Control Using Multiple Reinforcement Learning Agents (Vol. 33).

Delage, E., & Ye, Y. (2010). Accessibility support: If you experience accessibility issues with this file, report them here Distributionally Robust Optimization Under Moment Uncertainty with Application to Data-Driven Problems. Operations Research, 62(6), 1358–1376. https://www.jstor.org/stable/40792682

Dhanaraj, N., Nemlekar, H., Nikolaidis, S., & Gupta, S. K. (2025). Proactive Contingency-Aware Task Allocation and Scheduling in Multi-Robot Multi-Human Cells via Hindsight Optimization. IEEE Transactions on Automation Science and Engineering, 22, 13046–13060. https://doi.org/10.1109/TASE.2025.3546281

Gademann, A. J. R. M. noud, Van Den Berg, J. P., & Van Der Hoff, H. H. (2001). An order batching algorithm for wave picking in a parallel-aisle warehouse. IIE Transactions (Institute of Industrial Engineers), 33(5), 385–398. https://doi.org/10.1080/07408170108936837

Gao, R., & Kleywegt, A. (2023). Distributionally Robust Stochastic Optimization with Wasserstein Distance. Mathematics of Operations Research, 48(2), 603–655. https://doi.org/10.1287/moor.2022.1275

Haouassi, M., Kergosien, Y., Mendoza, J. E., & Rousseau, L. M. (2022). The integrated orderline batching, batch scheduling, and picker routing problem with multiple pickers: the benefits of splitting customer orders. Flexible Services and Manufacturing Journal, 34(3), 614–645. https://doi.org/10.1007/s10696-021-09425-8

Lamballais, T., Roy, D., & De Koster, M. B. M. (2017). Estimating performance in a Robotic Mobile Fulfillment System. European Journal of Operational Research, 256(3), 976–990. https://doi.org/10.1016/j.ejor.2016.06.063

Lenoble, N., Frein, Y., & Hammami, R. (2018). Order batching in an automated warehouse with several vertical lift modules: Optimization and experiments with real data. European Journal of Operational Research, 267(3), 958–976. https://doi.org/10.1016/j.ejor.2017.12.037

Liu, Z., Lu, J., Ren, C., Chen, J., Xu, Z., & Zhao, G. (2025). Joint optimization of storage assignment and order batching in robotic mobile fulfillment system with dynamic storage depth and surplus items. Computers and Industrial Engineering, 200. https://doi.org/10.1016/j.cie.2024.110767

Lu, M., & Shen, Z. J. M. (2021). A Review of Robust Operations Management under Model Uncertainty. Production and Operations Management, 30(6), 1927–1943. https://doi.org/10.1111/poms.13239

Ma, S., Ruan, J., Du, Y., Bucknall, R., & Liu, Y. (2025). An End-To-End Deep Reinforcement Learning Based Modular Task Allocation Framework for Autonomous Mobile Systems. IEEE Transactions on Automation Science and Engineering, 22, 1519–1533. https://doi.org/10.1109/TASE.2024.3367237

Mandi, J., Kotary, J., Berden, S., Bucarey, V., de, U. O., Leuven, K., & Ferdinando Fioretto, B. (2024). Benchmark and Future Opportunities Maxime Mulamba. In Journal of Artificial Intelligence Research (Vol. 81). https://github.com/PredOpt/predopt-benchmarks.

Mohajerin Esfahani, P., & Kuhn, D. (2018). Data-driven distributionally robust optimization using the Wasserstein metric: performance guarantees and tractable reformulations. Mathematical Programming, 171(1–2), 115–166. https://doi.org/10.1007/s10107-017-1172-1

Pardo, Gil-Borrás, Alonso-Ayuso, & Duarte. (2024). Order batching problems: Taxonomy and literature review. European Journal of Operational Research, 313(1), 1–24. https://doi.org/10.13039/501100011033

Qin, Z., Kang, Y., & Yang, P. (2024). Making better order fulfillment in multi-tote storage and retrieval autonomous mobile robot systems. Transportation Research Part E: Logistics and Transportation Review, 192. https://doi.org/10.1016/j.tre.2024.103752

Rasmi, S. A. B., Wang, Y., & Charkhgard, H. (2022). Wave order picking under the mixed-shelves storage strategy: A solution method and advantages. Computers and Operations Research, 137. https://doi.org/10.1016/j.cor.2021.105556

Richer, C., Bierlaire, M., & Torres, F. (2024). The Static Elevator Dispatching Problem with Destination Control. In Combinatorial Optimization and Applications. A Tribute to Bernard Gendron (pp. 311–343). Spring Nature. https://doi.org/10.1007/978-3-031-57603-4_14

Sadana, U., Chenreddy, A., Delage, E., Forel, A., Frejinger, E., & Vidal, T. (2025). A survey of contextual optimization methods for decision-making under uncertainty. In European Journal of Operational Research (Vol. 320, Number 2, pp. 271–289). Elsevier B.V. https://doi.org/10.1016/j.ejor.2024.03.020

Schiffer, M., Boysen, N., Klein, P. S., Laporte, G., & Pavone, M. (2022). Optimal Picking Policies in E-Commerce Warehouses. Management Science, 68(10), 7497–7517. https://doi.org/10.1287/mnsc.2021.4275

Scholz, A., Schubert, D., & Wäscher, G. (2017). Order picking with multiple pickers and due dates – Simultaneous solution of Order Batching, Batch Assignment and Sequencing, and Picker Routing Problems. European Journal of Operational Research, 263(2), 461–478. https://doi.org/10.1016/j.ejor.2017.04.038

Song, Y., & Luedtke, J. (2015). An Adaptive Partition-Based Approach for Solving Two-Stage Stochastic Programs with Fixed Recourse. SIAM Journal on Optimization, 25(3), 1344–1367. https://doi.org/10.1137/140967337

Tadumadze, G., Wenzel, J., Emde, S., Weidinger, F., & Elbert, R. (2023). Assigning orders and pods to picking stations in a multi-level robotic mobile fulfillment system. Flexible Services and Manufacturing Journal, 35(4), 1038–1075. https://doi.org/10.1007/s10696-023-09491-0

Tappia, E., Roy, D., De Koster, R., & Melacini, M. (2015). Modeling, Analysis, and Design Insights for Shuttle-Based Compact Storage Systems. SSRN Electronic Journal. https://doi.org/10.2139/ssrn.2631137

Tsai, I. N., Wu, Y. X., Huang, Y. H., Chen, Y. C., & Ding, J. J. (2025). Optimization of Elevator Standby Scheduling Strategy in Smart Buildings. Applied System Innovation, 8(5). https://doi.org/10.3390/asi8050132

Vera, A., Banerjee, S., & Gurvich, I. (2020). Online Allocation and Pricing: Constant Regret via Bellman Inequalities. http://arxiv.org/abs/1906.06361

Wan, J., Lee, K., & Shin, H. (2024). Traffic pattern-aware elevator dispatching via deep reinforcement learning. Advanced Engineering Informatics, 61. https://doi.org/10.1016/j.aei.2024.102497

Wang, R., Tao, P., Chen, R., Zhu, T., & Yang, P. (2025). Retrieval scheduling in four-directional shuttle-based compact storage and retrieval systems with heterogeneous lifts. Computers and Industrial Engineering, 210. https://doi.org/10.1016/j.cie.2025.111559

Wen, C., & Ma, H. (2024). An indicator-based evolutionary algorithm with adaptive archive update cycle for multi-objective multi-robot task allocation. Neurocomputing, 593. https://doi.org/10.1016/j.neucom.2024.127836

Wesselhöft, M., Hinckeldeyn, J., & Kreutzfeldt, J. (2022). Controlling Fleets of Autonomous Mobile Robots with Reinforcement Learning: A Brief Survey. Robotics, 11(5). https://doi.org/10.3390/robotics11050085

Wiesemann, W., Kuhn, D., & Rustem, B. (2013). Robust markov decision processes. Mathematics of Operations Research, 38(1), 153–183. https://doi.org/10.1287/moor.1120.0566

Wu, J., Yang, Z., Zhen, L., Li, W., & Ren, Y. (2025). Joint optimization of order picking and replenishment in robotic mobile fulfillment systems. Transportation Research Part E: Logistics and Transportation Review, 194. https://doi.org/10.1016/j.tre.2024.103930

Wu, Z., Zhang, Y., Li, L., Zhang, Z., Zhao, B., Zhang, Y., & He, X. (2024). Research on Inbound Jobs’ Scheduling in Four-Way-Shuttle-Based Storage System. Processes, 12(1). https://doi.org/10.3390/pr12010223

Žulj, I., Salewski, H., Goeke, D., & Schneider, M. (2022). Order batching and batch sequencing in an AMR-assisted picker-to-parts system. European Journal of Operational Research, 298(1), 182–201. https://doi.org/10.1016/j.ejor.2021.05.033